IALA Guideline

Gnnnn

Light Measurement

Edition x.x

Date (of approval by Council)

urn:mrn:iala:pub:gnnnn

Revisions to this document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |

1. Introduction	5

2. DEfinitions	5

3. Standard measurement conditions	5

3.1. measurement geometry	5

3.2. Ambient conditions	6

3.3. POWER SUPPLY CONDITIONS	6

4. Measurement preparation	6

4.1. Mounting	6

4.2. Warm-up	6

4.3. Measurement distance	6

5. Mesurement of marine signal lights	7

5.1. Luminous Intenisty versus angle	7

5.1.1. measurement method/procedure	7

5.1.2. Vertical Divergence	7

5.1.3. Horizontal Divergence	8

5.1.4. Specification Peak Intensity	8

5.2. Luminous intenisty versus time	8

5.2.1. mearement method/procedure	8

5.2.2. Flash Duration	8

5.2.3. Effective Intensity	8

5.2.4. Nominal Range	8

5.3. Colour and Sectors	8

5.3.1. mearement method/procedure	8

5.3.2. Signal Colour	9

5.3.3. Sector Colour Boundary	9

5.3.4. Sector Intensity Boundary	9

5.3.5. Sector Width	9

5.3.6. Sector Boundary	9

5.3.7. Sector of Uncertainty	9

6. abbreviations	9

7. references	9

8. Further reading	9

APPENDIX 1 Further Technical Guidance	10

1. The Measurement Laboratory	10

1.1. Stray and ambient light control	10

1.2. Folded Path Measurement	10

1.3. Zero Length Measurement	10

2. The Device Under Test	10

2.1. Reference center	10

2.2. SettingsSpectral correction	10

3. Photometry	10

3.1. Photometric distance law	10

3.2. Measurement by Photometer	10

3.3. Measurement by Spectroradiometer	10

3.4. Calibration by Substitution with Calibrated Light Source	10

3.5. Calibration using Known Distance	10

3.6. Relative Photometry of Optical Systems	11

3.7. Measurement of Luminous Flux	11

3.8. Measurement of Modulated Light	11

3.9. The convolution to determine flash duration	11

4. Colourimetry	11

4.1. Measurement by Spectroradiometer	11

4.2. Measurement by Tristimulus Colourimeter	11

5. Measurment in the Field	11

5.1. Mesurement of charater of AtoN light based on spectral analysis	11

5.2. Mesurement of AtoN light based on image processing technology	11

5.3. Mesurement based on prisms and prisim frame	11

List of Tables

Table 1 Example Elevation Profile Summary	8

List of Figures

Figure 1	X-Y Coordinate System	5

Figure 2	Example luminous intensity versus elevation profile	6

Introduction

This guideline supports IALA Recommendation R0203 Definitions of Marine Signal Lights Terms of Measurements [1]. The main body provides further explanations, diagrams and examples to the recommendations given in R0203. They will cover the measurements as well as both processing and presentation of results. These sections are intended for measurement operators with some experience.

An appendix contains further information including introductory topics, aimed at those newer to light measurement, and more detailed topics to assist readers in obtaining accurate and precise measurements. However, it is unfeasible for a single document to transfer all knowledge and skills required. It is recommended that measurement personnel complete dedicated training courses and read additional material.

DEfinitions

Standard measurement conditions

measurement geometry

R0203 recommends an X-Y coordinate system. This system is illustrated in Figure 1. The system has the following characteristics:

- The coordinates at the datum position are X = 0° and Y = 0°.

- The X coordinate is the rotation of the DUT about a vertical axis.

  - Rotating the DUT clockwise increases the value of X up to a maximum of 180°.

  - Rotating the DUT anticlockwise decreases the value of X down to a minimum of -180°.

  - The X angle is often referred to as the “horizontal” or “azimuth” angle since the rotation occurs in the horizontal plane.

- The Y coordinate is the rotation of the DUT about a horizontal axis that is perpendicular to the measurement path. Viewing with the measurement sensor to the right of the DUT as in Figure 1:

  - Rotating the DUT clockwise increases the value of Y up to a maximum of 180°.

  - Rotating the DUT anticlockwise decreases the value of Y down to a minimum of -180°.

  - The Y angle is often referred to as the “vertical” or “elevation” angle since the rotation occurs in the vertical plane.

- The axis of rotation and measurement path should intersect. The centre of the DUT should be positioned at this intersection.

X-Y Coordinate System

Ambient conditions

POWER SUPPLY CONDITIONS

Measurement preparation

Mounting

  - Settings

Warm-up

Measurement distance

Measurement of marine signal lights

Luminous Intenisty versus angle

measurement method/procedure

Vertical Divergence

R0203 gives the following definition:

“The average of all measured Full Width Half Maximum (FWHM) values shall be reported as the vertical divergence, along with the maximum deviation of the maximum intensity from an elevation of Y = 0°.”

These parameters are calculated from a light’s luminous intensity versus elevation angle profiles. An example profile plot, annotated with the FWHM, is shown in Figure 2.

An example summary of three elevation profiles is shown in Table 1. In this example three elevation profiles are summarised, each taken at a different azimuth coordinate. The first row of results corresponds to the profile shown in Figure 2; these should be compared to aid understanding.

The FWHM is the angle between the points where the plot crosses 50 % of the peak intensity. It is possible that the intensity of the plot drops below 50 % of the peak and then rises above, giving multiple crossing points. In this situation, the innermost angle on each side of the peak is used.

Table 1 shows the angle at which the maximum intensity occurs in each elevation profile. The angle with the greatest magnitude (positive or negative) is presented as the maximum deviation of the maximum intensity from an elevation of Y = 0°.

Example Luminous Intensity Versus Elevation Profile

Example Elevation Profile Summary

| Azimuth Measurement Angle (X) | FHWM | Angle of Maximum Intensity. |
| --- | --- | --- |
| -120° | 3.35° | -1.30° |
| 0° | 2.95° | -0.45° |
| 120° | 3.05° | 0.15° |
| Vertical Divergence | 3.12° |  |
| Maximum deviation from 0° |  | -1.30° |

Additional guidance to add:

Number of profiles to measure for: Omni Directional lantern, Sector Light, Directional/Rotating Light

Normalisation This profile has been normalised, to the peak of the beam. This normalisation is optional and each lighthouse authority may choose whether to present absolute intensity values, normalise and what to normalise to.

Horizontal Divergence

Specification Peak Intensity

Luminous intenisty versus time

mearement method/procedure

Flash Duration

Effective Intensity

Nominal Range

Colour and Sectors

measurement method/procedure

Signal Colour

Sector Colour Boundary

Sector Intensity Boundary

Sector Width

Sector Boundary

Sector of Uncertainty

abbreviations

DUT	Device Under Test

references

IALA. (2022) Recommendation R0203 Definitions of Marine Signal Lights Terms of Measurement.

Further reading

Any texts that are recommended to the reader without direct reference in the text should be listed within this section using the same syntax as the reference list. Sources should be listed using the Further reading style.

Einstein, A. (1905) Relativity: The Special and General Theory of Relativity

Idle, E. (1984) The Galaxy Song

Further Technical Guidance

The Measurement Laboratory

Overview

Stray and ambient light control

Folded Path Measurement

Zero Length Measurement

The Device Under Test

Reference center

Photometry

Photometric distance law

Measurement by Photometer

Spectral Correction Factor

Measurement by Spectroradiometer

Calibration by Substitution with Calibrated Light Source

Calibration using Known Distance

Relative Photometry of Optical Systems

Measurement of Luminous Flux

Measurement of Modulated Light

The convolution to determine flash duration

Colourimetry

Measurement by Spectroradiometer

Measurement by Tristimulus Colourimeter

Measurement Equipment Requirements

Photometer

Spectroradiometer

Colouromiter

Measurement in the Field

Measurement of character of AtoN light based on spectral analysis

Measurement of AtoN light based on image processing technology

Measurement of Vertical Profile Using prisms